3D Robotic Gripper Design & Analysis Report

Comprehensive AutoCAD Modeling Guide, Mechanics Calculations, & Component Specifications

1. Project Summary & Deliverables Matrix

This technical report documents the design, force analysis, actuator selection, and step-by-step AutoCAD modeling process for a robotic two-finger gripper system designed to manipulate a target cube.

| Deliverable Item | Requirement Description | Document Section / Guidance |
| --- | --- | --- |
| 1. Complete CAD Assembly | 3D CAD model containing a mounting plate, two fingers, sliding mechanism, actuator model, and target cube. | Section 4.2 (AutoCAD Modeling Steps) |
| 2. Labelled Drawing | 2D engineering drawing showing primary dimensions, stroke travel, and mounting hole pitch. | Section 4.4 (Layout & VIEWBASE Workflow) |
| 3. Grip-Force Calc & Components | Calculated minimum gripping force based on mass/friction and selected component list. | Section 2 & Section 3 (Calculations & BOM) |
| 4. Exploded View & Animation | Exploded assembly view and step-by-step setup for centered vs. off-center grasping motion. | Section 4.3 & Section 4.5 (Motion & Explosion) |

2. Mechanical Analysis & Grip-Force Calculation

2.1 Physics Model & Governing Equations

To prevent the target cube from slipping during robot manipulation, the friction force generated by the gripper fingers must overcome the gravitational pull as well as dynamic acceleration forces during movement.

Physics Model & Equation Definition F_grip = (S_f x m x g) / (n x μ) Where: • m = Mass of the Target Cube = 0.5 kg • g = Gravitational Acceleration = 9.81 m/s² • W = Weight of the Cube (m x g) = 0.5 kg x 9.81 m/s² = 4.905 N • S_f = Dynamic Safety Factor = 2.0 (accounts for robot acceleration, start-stop vibration, and dynamic loads) • n = Number of Contact Surfaces = 2 (two opposing fingers) • μ = Friction Coefficient = 0.30 (Aluminum/Dry Contact with rubber insert)

2.2 Numerical Calculation

Substituting the assumed design parameters into the friction model:

F_grip = (2.0 x 0.5 kg x 9.81 m/s²) / (2 x 0.30)

F_grip = 9.81 N / 0.60 = 16.35 N

Assuming a direct 1:1 mechanical advantage ratio through the linear sliding mechanism, the minimum linear force required from the actuator is F_actuator ≥ 16.35 N.

2.3 Actuator Selection & Verification

• Selected Component: Compact Pneumatic Cylinder (Standard ISO/SMC design, e.g., Festo ADN-16-25-A-P-A or SMC CQ2B16-25D)

• Cylinder Bore Diameter (D): 16 mm (0.016 m)

• Piston Area (A): (π x D²) / 4 = (π x 16²) / 4 = 201.06 mm²

• Standard Operating Pressure (P): 6 bar = 0.6 MPa = 0.6 N/mm²

• Output Force (F_actuator): P x A = 0.6 N/mm² x 201.06 mm² = 120.64 N

Design Justification Summary The selected 16 mm bore pneumatic cylinder delivers a theoretical output force of 120.64 N at standard 6 bar factory operating pressure. This provides an effective safety factor of approximately 7.38x over the minimum required gripping force of 16.35 N, guaranteeing secure holding under dynamic operations.

3. Bill of Materials (Component Selection)

Below is the short list of selected components making up the complete CAD assembly:

| Item No. | Component Name | Material / Specifications | Quantity |
| --- | --- | --- | --- |
| 1 | Mounting Plate | Aluminum 6061-T6 (100 x 100 x 10 mm) with standard mounting pitch | 1 |
| 2 | Linear Guide Rails & Slides | Stainless Steel miniature linear motion rails (Parallel mechanism) | 2 |
| 3 | Gripper Finger Blocks | Aluminum 6061-T6 with high-friction nitrile rubber pads | 2 |
| 4 | Pneumatic Actuator | Compact Double-Acting Cylinder (16 mm bore, 25 mm stroke) | 1 |
| 5 | Target Cube | 50 x 50 x 50 mm test block (Mass: 0.5 kg) | 1 |

4. AutoCAD 3D Modeling Step-by-Step Guide

4.1 Environment & Workspace Setup

1. Launch AutoCAD and choose the 3D metric template (acad3d.dwt).

2. Switch Workspace: Click the Gear Icon in the bottom-right status bar and select 3D Modeling.

3. Set View Orientation: Type VIEW in the command line, select SE Isometric, and click Apply.

4. Set Visual Style: Type VSMODE and choose Conceptual (or Realistic) for shading.

4.2 Modeling the Assembly Components

1. Mounting Plate:

• Command: BOX → Specify start point (0,0,0) → Type L (Length) → X = 100, Y = 100, Z = 10.

• Bolt Holes: Command CYLINDER → Center at (15,15,0) → Radius = 4, Height = 15. Repeat for corners (85,15,0), (15,85,0), (85,85,0).

• Cut Holes: Command SUBTRACT → Select the base plate → Press Enter → Select the four cylinders → Press Enter.

2. Actuator Model:

• Cylinder Body: Command CYLINDER → Base center (50,50,10) → Radius = 15, Height = 60.

• Piston Rod: Command CYLINDER → Base center (50,50,70) → Radius = 6, Height = 25.

3. Gripper Fingers (Opposing Pair):

• Left Finger: Command BOX → Start point (20,40,70) → X = 10, Y = 20, Z = 50.

• Right Finger: Command COPY → Select Left Finger → Copy to displacement position (70,40,70).

4. Target Cube:

• Centered Placement: Command BOX → Start point (37.5, 37.5, 90) → X = 25, Y = 25, Z = 25.

4.3 Motion Demonstration Setup (Centered vs. Off-Center Grasping)

• Centered Case: Move fingers inward symmetrically using the MOVE command along the X-axis (±10 mm displacement) until both rubber pads simultaneously contact the cube faces.

• Off-Center Case: Shift the target cube by +5 mm along the X-axis (Start point: 42.5, 37.5, 90). Move the fingers inward: observe how the right finger contacts the cube first, sliding it back to center alignment as the stroke completes.

• Recording Motion Animation: Use the ANIPATH command to define a camera path around the assembly, then record keyframe motion sequence as an MP4 or WMV video.

4.4 Generating 2D Labelled Drawing Layout (Deliverable 2)

1. Switch from Model Space to Layout1 tab.

2. Command: VIEWBASE → Select 'From Model Space'.

3. Place the Front View, Top View, Side View, and an Isometric View on the sheet.

4. Command: DIMLINEAR / DIMALIGNED → Add key dimensions (plate dimensions, mounting hole pitch, max finger stroke).

5. Command: MLEADER → Add leader lines and text callouts labeling the Mounting Plate, Pneumatic Actuator, Guide Rails, Finger Pads, and Target Cube.

4.5 Creating Exploded View Assembly (Deliverable 4)

1. In Model Space, offset components along their assembly motion axes using MOVE:

• Move Mounting Plate down along Z-axis by -30 mm.

• Move Actuator Body up along Z-axis by +20 mm.

• Move Left Finger outward along X-axis by -30 mm; Move Right Finger by +30 mm.

2. Command: LINE → Draw explosion trace lines (set linetype to Phantom or Dashed) connecting exploded components back to their assembly centers.